Program Arm1

- Install flash_loader_demo_v2.8.0.exe.

- Connect your scope to PC serial interface via RS232 to TTL module or USB to TTL module as picture1.

VCC does not need to be connected.

- Short circuit the two points in the following picture, and connect The device to computer via USB cable at the same time.

- Open the software STMFlashLoader Demo.

- Select the port and click “Next”.

- Click “Next”.

- Select the .bin file, “Erase necessary pages” and address (8000000). Click “Next ” to start.

- Disconnect the USB cable.

If the scope can't start normally, need to update the firmware(arm2) again.